Directnt, c't 1/97 S. 312, as , english

----------------------------------------------------------------

Programs for Windows NT 4.0

directnt.doc   ; This text

directnt.cpp   ; Listing of the driver, compilation needs DDK

directnt.sys   ; compiled Driver

directnt.h     ; Include file which specificates the Interface

directnt.rc    ; Ressource file

cx.cpp         ; Cyrix 6x86 configuration program: source

cx.exe         ; Cyrix 6x86 configuration program: executable german

cxe.exe        ; Cyrix 6x86 configuration program: executable english

showmem.cpp    ; Sample program for the use of Directnt

showmem.exe    ; executable german

showmeme.exe   ; executable english

setup.exe      ; Installer for the driver

setup.cpp      ; Source of the installer

setup.rc       ; Ressource file

inst.bat       ; Loads the driver

stop.bat       ; Unloads then  driver

m.bat          ; "Makefile" for Visual C++ 2.0 for directnt.sys

; as batch file (adjust paths!!!)

giveio.sys     ; Direct I/O from Dr. Dobb's 5/96

directio.zip   ; Sources and Readme from  Dr. Dobb's 5/96

; availabe in

; ftp://ftp.mv.com/pub/ddj/1996/1996.05/directio.zip

--------------------------------------------------------------

The driver Directnt.sys allows direct access to ports, PCI and the

internal processor registers CR0, MSR and to the memory (physically

addressed, only read access!). All the supported functions of the driver

are listed in the include file directnt.h

CX.exe is an application that communicates with the directnt driver.

You can activate (or deactivate) several features of the cyrix 6x86

processor:

Write Back or Write Through Cache Stragegy

Suspend on Halt (saving a lot of power)

Writegathering and Weak Write Ordering for the framebuffer

accelerating access to the framebuffer.

CPUID-Enable:  can be necessary, if software needs CPUID

(e.g. to detect MMX-Support).

Write-Allocation Strategy

Linear Burst (read only)

Parameter:

-----------

("Parameter: /WB :L1-Cache Write Back,   /WT : Write Through \n");

("           /SP :Suspend on HALT active,/NSP: passive\n");

("           /FF :Fast Framebuffer      ,/NFF: Slow\n");

("           /ID :CPUID on              ,/NID: CPUID off\n");

("           /WA :Write Allocation on   ,/NWA: Write Allocation off\n");

("           /Wait: Console waits for Key \n");

The application Showmem.exe is a sample for peeking into Windows

NT memory.

Showmem xxx yyy

shows the contents of the physical addresses from xxx to yyy.

Installation

-----------------------------------------------------

First of all, you have to install the driver with Setup.exe.

Then you must reboot the system. Now the driver is

automatically ready for use by applications.

Finally you can configure the processor with cx /xxx /yyy /zzz

It's useful to put CX with the parameters wanted into the startup folder.

If you like to see the message window of cx, you can add /wait,

then cx waits for a key stroke.

Stop.bat is a batch file for unloading the driver, with inst.bat you can

load it once more (adjust path!). This is very useful for testing new

driver versions.

Giveio.sys from Dale Roberts (Dr.Dobb's 5/96) can permit accesses to the

whole I/O-Area (even for a DOS-Box). But it is somewhat dangerous, because

usually NT has to survey many I/O-Accesses. You find more information

in the readme file in directio.zip!

Questions, suggestions, criticisms => as@ct.heise.de